CAJA FUSIBLES LADO VOLANTE

AV CONNECTOR (52 PINS AMP Multilock)

Pin Pin function

01 Low active analog input (not used)

02 Low active analog input (not used)

03 Windscreen Washer Pump external relay coil negative command

04 LIN serial line (not used)

05 LH front direction light actuation

06 CSG fault low active analog signal

07 Immobilizer antenna

08 Immobilizer antenna

09 Engine coolant level switch negative signal (N.O.)

10 VSO signal from NFR

11 Fog lamps relay coil negative command

12 High beam relay coil negative command

13 INT from F-37

14 LH wheel TPMS signal (not used)

15 RH wheel TPMS signal (not used)

16 Rear Window Washer Pump external relay coil negative command

17 30 from F-39 for NCL

18 Horn external relay coil negative command

19 Windscreen wiper cam negative signal

20 Headlamp washer relay coil negative command

21 Driver's side door open signal repetition

22 K line for NFR

23 FIS signal negative input

24 Negative analogical signal from brake fluid level switch (N.O.)

25 Alternator D+ positive signal

26 K line for NCM diagnosis

27 Ground reference for TPMS sensors (not used)

28 RH front direction light actuation

29 Windscreen Wiper Speed 2 relay coil negative command

30 Ground reference for RF antenna

31 DRL Lights Control

32 Negative signal from LH front brake pad wear sensor

33 Negative signal from bonnet open signalling switch (NC)

34 A-bus serial line on front cable for CSA

35 B-CAN A

36 B-CAN B

37 Tachometer 1 (VSO) signal repetition for CSG

38 LIN serial line (not used)

39 RH front side light actuation

40 Windscreen Wiper ON Speed 1 relay coil negative command

41 Rear Window Wiper relay coil negative command

42 LED Positive Command

43 Antenna for RF receiver

44 C-CAN L for NAS

45 C-CAN H for NAS

46 30 from F-39 for CSA

47 K line for CSG - CRS - NCA

48 C-CAN L

49 C-CAN H

50 LH front direction light actuation

51 RH front direction light actuation

52 LH front side light actuation